The S10 4x4 vacuum thread.

A lot of people have problems with their 4x4 not working, for various reasons. One of the common reasons is a bad vacuum actuator or bad vacuum lines going to it. 

In S10's, when you put it in 4WD (4 HI or 4 LOW) with either the 233 (push button) or 231 (manual shift(also 207)), the transfer case has a switch of sorts that tells the front axle to engage. This switch routes vacuum to one of two places. If you are in 4WD (HI or LOW), the vacuum goes to the actuator. If you are in 2 HI then the vacuum gets routed to an orb where it is just stored. This line also supplies the vacuum. 

The switch itself is on the driver's side, almost at the front of the t-case. It has three vacuum lines going to it. The switch works similar to a ball point pen, meaning there is a round ball that is either pushed or not. The ball gets pushed by the shift fork inside the t-case, when the case is shifted into 4wd. There are three vacuum lines going to the switch. One line supplies the vacuum and stores it in the orb when not in 4wd. One line vents to the open air and the t-case vent piggy backs onto it. The other line goes to the actuator. All three lines run together up to the passenger side of the bell housing, then two go one way and one goes the other.  The vacuum supply/orb is the line that breaks from the pack and follows over to the driver's side. The other two lines, the vent and the actuator line, continue to run along the passenger side frame rail where they end near the vacuum actuator. One line plugs into the actuator and the other is just a vent that clips onto the inside of the outer fender well. 

The actuator takes the vacuum, when supplied, and has a diaphragm that pulls on a cable. This cable runs down to the passenger side of the front differential and moves a collar back and forth. This collar connects and disconnects the passenger side cv shaft to the carrier. This is how the 4wd system works on an S10 and all IFS 4x4 GM products (with a few variants). 

This is the switch installed on a 231. Note the vent line coming off the t-case:

This is where the t-case vent line meets up with the open air circuit.

The lines running along the trans and bell housing.

This line goes off to the orb and supplies the vacuum from the engine:

This line follows the wiring harness to the orb, located behind the driver's side headlight:

This is the orb, aka the vacuum canister:

This is where the switch gets vacuum, off the intake:

These are the other two lines that go off to the passenger side:

They end behind the passenger side headlight:

This is the line going into the actuator:

The actuator is under the battery tray:

The t-case vacuum switch looks like this:

Use a 7/8" wrench to remove.  MAC Tools DA 28 Angle Wrench fits.

If you look at the top, you will notice a little "key":

This lines up with a similar spot on the female end:

The three vacuum lines must be in the right spot in order to function properly. Look at the female end of the vacuum lines with the open end facing towards you like in the above pic.  If you orientate it to where the key is on top, the lines are as follows:

Get a helper, a vacuum gage, mity vac pump, long screwdriver, pen and paper, some short pieces of vacuum hose, a short 7/8" wrench, ratchet, extension, 12 and 15 mm deep sockets, 5/16" wrench, (2) 24" battery cables (or a jump box), electrical tape, (2) 5/16" x 2" bolts and nuts, a catch pan, adjustable wrench, leak detector, Simple Green, and a headlight.  Remove the battery and tray.  Set the battery on top of the radiator.  Bolt the short battery cables to the truck's cables, wrap the positive connection, then connect them to the battery.  Now the truck engine will start and run so diagnostics can be performed.  Disconnect the vacuum hose from the actuator and attempt to operate it with the Mity Vac.  It should move with little resistance.  Mine operated slowly with 7 inches of vacuum.  Have your helper start the truck engine and engage 4WD, and report when the 4WD indicator lights ("lit" or "dark").  Use a vacuum gage to check engine vacuum, then compare that to the vacuum at the canister and the actuator.  Record the vacuum readings to shorten diagnostic time and to compare after a hose or part is replaced.  If they are not equal, start at the engine and follow the vacuum hoses out, testing after each connection and spraying leak detector on each hose.  I found a small vacuum leak in the hose going to the dash, but the large leak was in the vacuum hose going from the engine to the T-case vacuum switch.  Go underneath the truck, remove the skid plate, and check for vacuum leaks with leak detector.  Disconnect the T-case vacuum hose connector from the vacuum switch with the long screwdriver & read the supply hose vacuum.  There is enough slack in the hoses to pull it down a few inches and should read the same as engine vacuum.  If trans fluid leaks out of the connection, shut off the engine and clean the vacuum hoses.  Spray Simple Green down from the top and blow it through the hose.  Remove the vacuum switch & connect it to the vacuum connector.  Push in the ball and listen for a vacuum leak.  No leaks, reinstall the switch & connector.  Check it for a vacuum leak by shifting back and forth from 2WD to 4WD.  I installed a new switch and had to replace it with the old one because it had a vacuum leak.  Check the transfer case fluid level.  Have a catch pan underneath because it is probably overfull.  Drain the transfer case to the full level.  To feed the vacuum lines, cut the head off of a 1/8" x 2" bolt.  Put one inch of the bolt in the old hose and one inch in the new hose.  Clean both hoses, then tightly wrap 100 mph tape around the connection.  Feed it down from the top while pulling from the bottom.  Push the taped connection thru the connector from the top with a long screwdriver.  Pull it down and match the OEM hose routing.  Open the clamp on the transmission & pull the hoses over it to the transfer case.  Replace one hose at a time and use the diagram.

The actuator vacuum line is almost 9' and the main vacuum line is almost 5'.  Both are 5/32".  The vent line is 7/32" and about the same length as the actuator line.  I did not remove it.

I have a '93 S10 Blazer Vortec with I'm assuming 233 (electronic 4wd). It recently quit working and left me stranded in mud. The button engages and it has the sound but nothing happens as far as front diff engaging. Is there a way to diagnose and narrow down what vacuum area it could be?

This is how to help narrow it down a bit. Take a set of vice grips and manually pull the cable at the actuator to take the vacuum completely out of the loop.  If the 4wd system does not work after this, then your problem is in the electronics, not the vacuum. If it works, then your problem is vacuum. To start diagnosing the vacuum system, make sure the lines going back to the t-case are intact and plugged in. If they are, then start the engine and make sure one of the lines will pull a vacuum. If it does, make sure its plugged into the valve properly. If that is correct then put it in 4wd (still with the motor running) and see if the line at the actuator is pulling a vacuum. This will narrow down which of the two lines may have a fault in it. The open air line does not matter, it could be un-plugged at the t-case for all it matters.

How to tell which vacuum line goes where if they are disconnected:

Remove the switch from the transfer case.  With the switch loose, meaning not pressed, one port is plugged, that is the one that gets vacuum.  Then with the switch pushed, a different port is plugged, that is a vent to release the vacuum and by default the third port goes to the front diff actuator.

Ok, the Vacuum ball usually just has one hose attached to it. From the back of the throttle body or intake port a 7/32 vacuum hose comes off and goes to the check valve (large end) Off the back (tee) it may branch off and go to another tee. One of these hoses off this tee would go over to the vacuum ball. One will go over to the cruise control small port on the servo. One port will go down to the transfer case vacuum switch, and one will go to the little black plastic line going inside the cab for the vacuum feed for the heater control.

None of these hoses that come off from behind the check valve go to any emission controls, they are strictly for accessory controls. The check valve keeps the vacuum in the storage ball during hard acceleration. So if your heater switches from vent to defrost during hard acceleration, replace this check valve. These hoses are not shown on the emission label diagram.

Here is a picture of the check valve. As long as the vacuum tank, cruise control servo, heater control and transfer case hoses are attached to the (TEE ) side of the check valve you should be good. You just have to follow the hoses to see where they connect to the components.

Underdash vacuum line routing: