Viper 211HV Installation Notes

1986 Chevrolet K5 Blazer with Power Locks

| Viper Control Module Wiring | Truck Wiring | Instructions / Notes |
| --- | --- | --- |
|  | 15A Fuse to Battery Voltage at Auxiliary Fuse Panel | Before Installation - Test door lock relay for proper operation and at least 15 amps of power - Ensure proper operation before installing Viper module Door Lock Motors - + to Gray locks the motors - + to Tan unlocks the motors - Opposite wire grounds Door Lock Relay - Relay is mounted to the brake pedal support under the dash - 2 Gray wires to lock/unlock motors - 2 Tan wires to lock/unlock motors - Orange/black wire is power - 2 Light Blue wires (lock signal from switches) - 2 Black/white wires (unlock signal from switches) - Cut one Light Blue one Black/white wire - Connect the Viper between the cuts using the chart here. - This method will unlock both doors with no second unlock output. To unlock passenger door separately, connect the Blue Viper wire to the truck’s gray passenger unlock wire. The green/black Viper wire will go to the truck’s gray driver unlock wire. |
|  | Not Used |  |
|  | Grounded |  |
|  | Splice into the White Dome Light Ground Wire from Driver’s Door |  |
|  | Relay Side of Door Lock Wire (Cut Light Blue Wire) |  |
|  | Switch Side of Door Lock Wire (Light Blue Wire Hot with Active Switch) |  |
|  | 15A Fuse to Battery Voltage |  |
|  | Relay Side of Unlock Wire (Cut Black/White Wire) |  |
|  | Spliced to Violet/Black (Battery Voltage) |  |
|  | Parking Lights—Splice into Brown Wire from Dash Light Switch |  |
|  | Ground |  |
|  | Horn-Black Wire from Steering Column Grounds Relay |  |
|  | Not Used |  |
|  | Rear Window Up-Use Relay to Tan/White wire at window switch |  |
|  | 15A Fuse to Ignition Voltage |  |
|  | Not Used |  |
|  | Switch Side of Unlock Wire (Black/White Hot with Active Switch) |  |
|  | Rear Window Down-Use Relay and Splice into Light Blue Wire at front window switch |  |